LUX  Install instructions

Caution:  Do at your own risk – I will not be held responsible for any malfunctions, injuries, or anything else that goes wrong.  

Overview of steps

Remove wheel

Remove wheel well

Remove OEM bulb

Install LUX 2.0

Put everything back together 

Tools needed (image 1-tools)

17mm Socket and wrench – Tire removal

10mm socket, 8 mm socket

Slotted screw Driver

T-15 Torx screw driver

Steps

Start of the Left side – that is where all the pictures were taken

1 – It is much easier to remove the wheel rather than trying to turn the wheel in different directions.  Jack the car up and use the 17mm socket to remove the nuts.   The wheel may be very hard to get off due to rust, and if so just keep pulling.  You may need to hit the inside of the wheel with a rubber mallet, and if you do make sure someone is watching the wheel so it doesn’t fall off and get damaged.  Place the wheel Face UP.

2 – Remove wheel well.

In total you will need to remove 9 screws and 1 nut.  Pictures 2-4 have all of the places circled.  Inside the wheel well there are 5 screws and 1 nut to remove.  On the bottom there are 4 screws to be removed.  In picture 4 there are 3 X’s.   Some people say you need to remove the screws / plug that are there, but you don’t need to, nor does it make anything easier.

Once all 10 items are removed, just wiggle the wheel well out and you will have access to the headlight, fog light, and everything else.  It may be easiest to remove the wheel well from the bottom first and then slide the top down.  Remember how this went in because you need to put it back the same way.

3 – Remove the OEM bulb.

For the rest of the pictures I took them from a extra headlight so the angle you will be viewing might be slightly different, but yet it is the exact same headlight

Picture 5 – This is the cover you want to remove.  There are 2 clips, one on top and one on bottom.  Use the slotted screw driver to unclip these.  TIP – Picture 6 shows you twisting the screw driver rather than prying.  If you twist the screw driver and gently pull on the cover you will feel it come loose.  Do this for the Top and Bottom

Picture 7 shows what the OEM bulb will look like.  Remove the 2 wires by pulling straight back, shown in picture 8.  Once the wires are removed we can unscrew the oem bulb using the T-15 Torx wrench, Picture 9.  Now see picture 10.  You will now have the OEM bulb and you will see the metal piece holding the clear glass.  Remove the metal piece ( and there is a plastic holder attached as well).

Next follow the progression as in picture 11.  The metal piece will NOT be used, but KEEP this and the OEM bulb just in case you need to switch back to stock.  Test fit the LUX 2.0 kit into the plastic holder ( don’t worry, it will fit, I tested them all).

Each LUX 2.0 unit came with 8 small screws.  You will slide 4 screws onto the oem bolt, as shown in picture 11 at the bottom.  These 4 screws are used just as spacers so the LUX 2.0 kit stays tight and you do not over tighten the unit and crack the headlight housing .

Now take the LUX 2.0 kit, the 2 screws (each with 4 spacers on them) and that plastic holder back to the car to install everything.

First, put the plastic holder back into the headlight with the 2 pins on top, shown in Pic 12. Now you will need to install the LUX 2.0 kit.  The fan goes on the bottom and as shown in pic 13, the 2 pins will slightly come through the top hole.  It is a little tricky getting the screws back in, but take your time.  Tighten the screws by hand, but make them very firm.

Connection the wires.  This is the most important piece of the whole instructions.

Green wire goes to Green wire

Black wire goes to Brown Wire

CAUTION – If you hook these wires up wrong, aka backwards, you will blow the power unit almost instantly, and then you’re… screwed.

So, after you connect the wires then you can test the kit out.  You don’t have to worry about the negative terminal touching anything because that will not affect anything.  The positive terminal ( the green-red one) is protected)

Just turn your parking lights on and make sure that the light is working and that it looks good.

After your satisfied, it’s time to put the cover back on.  As shown in pic 15 the cover has a top and bottom.  The bottom needs to go where the fan is, otherwise it will not fit. Pic 16 shows how to re-install the over back onto the headlight.  You will hear 2 little clicks once the top and bottom clips have snapped into place.

Now all that’s left is putting everything back together and going on to the other side.

If you have any questions feel free to e-mail me at mds82@yahoo.com or check the forum because there is a lot of info in the DIY section.

ENJOY